NAME: NISHEVITHA SRI.S

REG NO: 192511424

COURSE: COMPUTER ARCHITECTURE

COURSE CODE: CSA1204

Q1. Compare L1, L2 and L3 Cache Levels

Cache memory is a high-speed memory located close to the processor. Modern processors generally use multiple cache levels: L1, L2 and L3. They differ mainly in capacity, latency, bandwidth and their impact on processor performance.

| Feature | L1 Cache | L2 Cache | L3 Cache |
| --- | --- | --- | --- |
| Location | Closest to CPU core | Near CPU core | Shared among cores |
| Size | Smallest | Medium | Largest |
| Latency | Lowest | Higher than L1 | Higher than L2 |
| Bandwidth | Highest | High | Lower than L1/L2 |
| Access speed | Fastest | Fast | Comparatively slower |
| Purpose | Frequently used data/instructions | Backup for L1 | Shared backup for L1/L2 |

L1 Cache

L1 cache has the lowest access latency and is usually divided into instruction cache and data cache. Since it is very close to the processor core, frequently accessed instructions and data can be retrieved quickly.

L2 Cache

L2 cache is larger than L1 but has slightly higher latency. If required data is not available in L1, the processor checks L2.

L3 Cache

L3 cache is generally larger and may be shared by multiple processor cores. It has higher latency than L1 and L2 but reduces the need to access slower main memory.

Impact on Processor Performance

A high cache hit rate reduces the number of accesses to main memory. This reduces average memory access time and improves processor throughput.

Conclusion:
L1 provides the fastest access, L2 provides a larger secondary cache, and L3 provides a large shared cache. The hierarchy improves processor performance by keeping frequently required data closer to the CPU. The assessment specifically requires comparison of these levels using latency, bandwidth and throughput.

Q2. Compare SRAM and DRAM

SRAM (Static RAM) and DRAM (Dynamic RAM) are two important types of memory used in computer systems.

| Feature | SRAM | DRAM |
| --- | --- | --- |
| Full form | Static Random Access Memory | Dynamic Random Access Memory |
| Latency | Very low | Higher than SRAM |
| Speed | Faster | Slower |
| Bandwidth | High | High, but generally lower effective access speed |
| Throughput | High for low-latency access | High for large-capacity transfers |
| Cost | Expensive | Less expensive |
| Density | Lower | Higher |
| Refresh | Not required | Required |
| Main use | Cache memory | Main memory |

SRAM

SRAM stores data using flip-flop circuits and does not require periodic refreshing. Its low latency makes it suitable for processor cache memory.

DRAM

DRAM stores data using capacitors and requires periodic refreshing. It provides higher storage density at lower cost, making it suitable for main memory.

In Cache and Main Memory

SRAM is preferred for cache because the processor needs very fast access to frequently used data.

DRAM is preferred for main memory because computers require larger memory capacity at reasonable cost.

Conclusion

SRAM provides lower latency and is suitable for cache memory, while DRAM provides higher capacity and is suitable for main memory. Therefore, both technologies play different roles in the memory hierarchy.

Q3. Virtual Memory with Paging vs Cache Memory

Virtual memory and cache memory are different mechanisms used to improve computer-system operation.

| Feature | Virtual Memory with Paging | Cache Memory |
| --- | --- | --- |
| Main purpose | Extends available memory using secondary storage | Speeds up CPU memory access |
| Storage | RAM + secondary storage | Small high-speed memory |
| Latency | High when page fault occurs | Very low on cache hit |
| Speed | Slower | Faster |
| Throughput | Lower during page faults | Higher for frequently accessed data |
| Management | Operating system | Hardware/cache controller |
| Unit | Pages | Cache blocks/lines |

Virtual Memory with Paging

Virtual memory allows a program to use an address space larger than the available physical RAM. Memory is divided into fixed-size pages.

When a required page is not present in RAM, a page fault occurs and the operating system loads the page from secondary storage.

This provides greater memory capacity but page faults can significantly increase access latency.

Cache Memory

Cache stores frequently accessed data and instructions close to the processor. When the required data is found in cache, it is accessed much faster than from main memory.

Throughput Comparison

Cache memory improves throughput by reducing the average time required for memory accesses.

Virtual memory improves the ability to run large programs but can reduce system performance when frequent page faults occur.

Conclusion

Cache memory primarily improves access speed and processor throughput, whereas virtual memory with paging primarily provides memory capacity and address-space flexibility. The assessment requires comparison of these mechanisms specifically in terms of access latency and system throughput.

Q4. NAND Flash vs DRAM

NAND Flash and DRAM are different memory technologies with different performance characteristics.

| Feature | NAND Flash | DRAM |
| --- | --- | --- |
| Type | Non-volatile | Volatile |
| Latency | Higher | Lower |
| Speed | Slower for random access | Faster |
| Bandwidth | High for sequential operations | High |
| Refresh | Not required | Required |
| Data retention | Retains data without power | Loses data without power |
| Main use | SSDs, storage | Main memory |
| Hierarchical level | Secondary/storage level | Main-memory level |

NAND Flash

NAND Flash is non-volatile memory. It retains stored information even when power is removed.

It is widely used in SSDs and other storage devices. It has much higher latency than DRAM, especially for random access, but provides persistent storage.

DRAM

DRAM is volatile memory and is normally used as the system's main memory. It provides much faster access compared with NAND Flash.

DRAM is therefore suitable for storing actively running programs and data.

Suitability in Hierarchical Memory

A typical hierarchy can be represented as:

CPU

↓

Cache

↓

DRAM

↓

NAND Flash / SSD

↓

Long-term Storage

DRAM is positioned closer to the processor because of its lower latency, while NAND Flash is used at the storage level because of its non-volatility and high capacity.

Conclusion

DRAM is suitable for high-speed main-memory operations, while NAND Flash is suitable for persistent storage. NAND Flash offers non-volatility, whereas DRAM provides significantly faster access.

Q5. Compare MRAM and RRAM

MRAM (Magnetoresistive RAM) and RRAM (Resistive RAM) are advanced non-volatile memory technologies designed to provide high performance with lower energy consumption.

| Feature | MRAM | RRAM |
| --- | --- | --- |
| Full form | Magnetoresistive RAM | Resistive RAM |
| Data storage | Magnetic state | Resistance state |
| Volatility | Non-volatile | Non-volatile |
| Access time | Fast | Fast |
| Throughput | High | High |
| Energy efficiency | High | High |
| Data retention | Good | Good |
| Potential use | Embedded memory, cache-like applications | Embedded memory, storage and memory systems |

MRAM

MRAM stores information using magnetic states. Since it is non-volatile, data can remain stored even when power is removed.

MRAM offers fast access and good endurance, making it suitable for applications that require frequent read/write operations.

RRAM

RRAM stores information by changing the resistance of a memory cell between different states. It is also non-volatile and can potentially provide high density and low-power operation.

Performance Comparison

MRAM generally provides fast access and good endurance, while RRAM offers the potential for high density and efficient memory implementation. Both technologies can reduce energy consumption compared with continuously powered volatile memory in suitable applications.

Conclusion

MRAM and RRAM are promising non-volatile memory technologies. Both provide fast access and good energy efficiency, but their underlying storage mechanisms are different. Their suitability depends on requirements such as access time, throughput, density and energy consumption. The assessment specifically asks for comparison of these technologies using access time, throughput and energy efficiency.