SIMATS ENGINEERING

Department of Computer Science & Engineering  ASSESSMENT TOOL 2-COMPARATIVE ANALYSIS

| Course Code: | CSA12 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO4 | Assessment: | ASSESSMENT TOOL2- COMPARATIVE ANALYSIS |
| Weightage: | 30% | Total Marks: | 25 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | Nishevitha sri.s | Reg.No.: | 192511424 |
| Department: | CSE | Date: |  |

ANNEXURE A

COMPARATIVE ANALYSIS

1.Compare L1, L2, and L3 cache levels in terms of latency, bandwidth, and throughput, and analyze their  impact on processor performance.

2.Differentiate SRAM and DRAM by evaluating their latency, bandwidth, and throughput characteristics  in cache and main memory design.

3.Compare virtual memory with paging and cache memory mechanisms in terms of access latency and  system throughput.

4.Analyze the differences between NAND Flash and DRAM with respect to latency, bandwidth, and  suitability for hierarchical memory systems.

5.Compare MRAM and RRAM technologies by examining their performance in terms of access time,  throughput, and energy efficiency.

Code of Conduct:

I Nishevitha sri.S(Name / Reg No) certify that this submission is my original  work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

# MARKS ALLOCATION

|  | Scope of Items (20%) | Comparison Depth (25%) | Use of Evidence (20%) | Critical Thinking (20%) | Clarity of Presentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Scope of Items | 20% | Selects highly relevant items with clear scope and justification | Selects appropriate items with minor gaps | Selection is limited or partially relevant | Items are unclear or not relevant |
| Comparison Depth | 25% | Provides detailed and comprehensive comparison across multiple aspects | Provides adequate comparison with some depth | Limited comparison with few aspects | Very minimal or no comparison |
| Use of Evidence | 20% | Supports comparison with accurate data, examples, or references | Uses some supporting evidence with minor gaps | Limited or weak supporting evidence | No supporting evidence provided |
| Critical Thinking | 20% | Demonstrates strong critical thinking with clear interpretation and reasoning | Shows reasonable interpretation with minor gaps | Basic interpretation; lacks depth | No meaningful interpretation |
| Clarity of Presentation | 15% | Well-organized, clear, and logically structured presentation | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |

# Question 1: L1, L2, and L3 Cache Comparison

Compare L1, L2, and L3 cache levels in terms of latency, bandwidth, and throughput, and analyze  their impact on processor performance.

# Architecture Diagram

# Scope of Items Compared

The comparison scope covers the three on-chip cache levels found in virtually all modern multicore  processors, evaluated across three performance dimensions relevant to CO4: access latency,  bandwidth, and throughput. These items are chosen because they directly determine how close the  memory subsystem comes to matching core execution speed — the central design tension in cache  hierarchy engineering.

# Comparison Table

| Parameter | L1 Cache | L2 Cache | L3 Cache |
| --- | --- | --- | --- |
| Typical Size | 32-64 KB (per core, split I/D) | 256 KB-1 MB (per core/cluster) | 2-32 MB (shared, last- level) |
| Access Latency | 1-4 cycles | 8-15 cycles | 30-50 cycles |
| Bandwidth | Highest (multiple ports) | High, shared port limits concurrency | Lower per-request, aggregated across cores |
| Parameter | L1 Cache | L2 Cache | L3 Cache |
| Throughput Role | Sustains near-peak IPC | Absorbs without DRAM penalty L1 misses | Absorbs cross-core misses, cuts DRAM traffic |
| Scope | Private, per-core | Private or per-cluster | Shared, whole-chip |

# Analysis 1 — Architectural / Structural Lens

Viewed structurally, the three levels form a strict inclusion pyramid built around physical  proximity to the execution pipeline. L1 sits adjacent to the ALUs and is split into instruction and  data banks so both can be fetched in the same cycle window; this physical closeness, not just size,  is what buys its 1-4 cycle latency. L2 trades that proximity for capacity, typically serving one core  or a small cluster, while L3 is architected as a distributed, banked, last-level structure shared across  the whole chip so that any core can observe data cached by another — essential for cache-coherent  multicore correctness.

# Analysis 2 — Quantitative / Data-Driven Lens

The numbers scale in a roughly geometric progression: each level is about 4-8x larger and roughly  3-4x slower than the one above it. Published Intel Core and AMD Zen data place L1 hit latency  near 4-5 cycles, L2 near 12-14 cycles, and shared L3 in the 30-40+ cycle range, with L1 per-core  bandwidth in the hundreds of GB/s versus L3's aggregate bandwidth being contended for by every  core on the die. This quantitative gap is precisely why a single L1 miss serviced by L2 costs  roughly 3-4x the L1 latency, and an L3 miss serviced by DRAM costs an order of magnitude more  again.

# Analysis 3 — Application / Critical-Thinking Lens

From a system-design standpoint, the three-way trade-off is fundamentally size versus speed  versus sharing scope. L1 cannot hold enough data to avoid frequent misses on its own, so it  depends on L2 as a low-penalty backstop; L3, being large enough to capture cross-core sharing  patterns, is valuable for multicore workloads with shared data structures but too slow to be relied  upon directly by the pipeline. This is why hierarchy-level engineering choices —  inclusive/exclusive policies, replacement algorithms, prefetching — tend to move real-world IPC  more than raw clock-frequency increases in most modern workloads.

# Synthesized Conclusion (Cross-Analysis Summary)

Taken together, the structural, quantitative, and applied perspectives converge on the same  conclusion: cache hierarchy performance is not a property of any single level but of how well the  levels compound to hide DRAM latency from the core. The structural view explains why the  latency gap exists (proximity and sharing scope), the quantitative view shows how large that gap  actually is in cycles, and the applied view shows why it matters — hierarchy design choices  dominate real-world IPC more than clock speed. Any comparative claim about cache performance  should therefore be read as a statement about the whole pipeline, not an isolated level.

# Question 2: SRAM vs DRAM

Differentiate SRAM and DRAM by evaluating their latency, bandwidth, and throughput characteristics in cache and main memory design.

# Architecture Diagram

# Scope of Items Compared

The scope compares SRAM and DRAM as the two dominant volatile memory technologies used  respectively for on-chip caches and main memory, examined on latency, bandwidth, throughput,  and the underlying circuit-level reasons (cell structure, refresh requirement) that justify why the  industry uses SRAM for cache and DRAM for main memory rather than either technology alone.  Comparison Table

| Parameter | SRAM | DRAM |
| --- | --- | --- |
| Cell Structure | 6-transistor capacitor (6T) flip-flop, no | 1 transistor + 1 capacitor (1T1C) |
| Latency | ~1-2 ns, no refresh | ~10-20 ns plus periodic refresh stalls |
| Bandwidth | High per bit, low density limits total capacity | Lower per-cell speed, wide buses give high aggregate bandwidth |
| Throughput | Near-every-cycle access for hits | Row-buffer/bank-conflict sensitive |
| Parameter | SRAM | DRAM |
| Density / Cost | Low density, high cost per bit | High density, low cost per bit |
| Typical Use | L1/L2/L3 cache, register files | Main system memory (DDR4/DDR5, LPDDR5) |

# Analysis 1 — Architectural / Structural Lens

At the circuit level, SRAM's 6-transistor bistable cell holds its state as long as power is applied,  requiring no refresh logic and enabling deterministic, sub-nanosecond-class access. DRAM's  1T1C cell stores a bit as charge on a capacitor, which leaks over time and must be refreshed roughly  every 64 ms — this refresh circuitry is a structural addition that SRAM simply does not need, and  it is the direct cause of DRAM's variable access latency.

# Analysis 2 — Quantitative / Data-Driven Lens

SRAM's 6-transistor cell is roughly 6-10x larger per bit than DRAM's 1-transistor-1-capacitor cell,  which inversely predicts their observed density and cost figures — SRAM cache capacities top  out in the tens of megabytes while DRAM main memory routinely reaches tens of gigabytes on  the same die-area budget. Latency figures track this inversely too: SRAM's ~1-2 ns versus  DRAM's ~10-20 ns is roughly a 10x gap, consistent with standard Hennessy & Patterson reference  figures for cache versus main-memory access time.

# Analysis 3 — Application / Critical-Thinking Lens

The practical consequence is a clean division of labor: caches must be small but fast enough to  keep pace with a multi-GHz core, so they are built from SRAM; main memory must be large  enough to hold entire program working sets, so it is built from DRAM despite its refresh overhead.  The cache hierarchy that sits between them exists specifically to hide DRAM's latency and refresh- related throughput variability from the pipeline — meaning SRAM and DRAM are not competing  technologies but complementary ones, each optimized for a different point on the size-versus- speed curve.

# Synthesized Conclusion (Cross-Analysis Summary)

The three lenses reinforce a single causal chain: a circuit-level difference (refresh requirement)  produces a quantitative gap (roughly 10x in latency, 6-10x in cell area) that in turn dictates an  architectural division of labor (SRAM for cache, DRAM for main memory). No comparison of  SRAM and DRAM is complete without tracing this chain — evaluating latency numbers alone,  without the underlying cell-structure reason, would miss why the technologies are deployed at  their respective tiers rather than substituted for one another.

# Question 3: Virtual Memory with Paging vs Cache Memory

Compare virtual memory with paging and cache memory mechanisms in terms of access latency and system throughput.

# Architecture Diagram

# Scope of Items Compared

The scope compares two structurally analogous memory-management mechanisms: cache  memory (a hardware-managed, transparent speed buffer between core and DRAM) and virtual  memory with paging (an OS/hardware-cooperative mechanism mapping a large virtual address  space onto limited physical DRAM, backed by disk/SSD). Both rely on locality, hit/miss behavior,  and replacement policy, making them directly comparable on access latency and system  throughput despite very different scales.

# Comparison Table

| Parameter | Cache Memory | Virtual Memory (Paging) |
| --- | --- | --- |
| Managed By | Hardware (transparent) | OS + hardware (MMU, page tables) |
| Hit Latency | 1-40 cycles (L1-L3) | ~100-300 cycles for TLB hit, in-memory page access |
| Miss Penalty | ~100-300 cycles (DRAM access) | Page fault: milliseconds (disk-backed) — 5-6 orders of magnitude slower |
| Parameter | Cache Memory | Virtual Memory (Paging) |
| Granularity | Cache line (32-128 bytes) | Page (4 KB, or 2 MB/1 GB huge pages) |
| Purpose | Hide DRAM latency from the core | Extend addressable memory; enable process isolation |
| Throughput Impact | Directly determines effective IPC | Page faults degrade throughput; TLB misses add page-walk overhead |

# Analysis 1 — Architectural / Structural Lens

Structurally, both mechanisms place a small, fast structure in front of a larger, slower one — a  cache line in front of DRAM, a TLB entry and page table in front of disk/SSD — and both depend  on locality of reference to keep the fast path in frequent use. The key architectural difference is  who manages the mechanism: cache replacement is entirely hardware-controlled and invisible to  software, whereas paging is a cooperative hardware/OS mechanism where the MMU walks page  tables that the operating system maintains.

# Analysis 2 — Quantitative / Data-Driven Lens

The latency gap between the two failure modes is stark: a cache miss serviced from DRAM costs  roughly 100-300 cycles (on the order of 100 ns), while a page fault serviced from disk/SSD costs  milliseconds — a difference of 5 to 6 orders of magnitude. Even on the hit path, a TLB hit followed  by an in-memory page access (~100-300 cycles) is already comparable to a cache miss penalty,  meaning paging's 'fast path' is roughly as expensive as caching's 'slow path.'

# Analysis 3 — Application / Critical-Thinking Lens

Because page faults are exceptional, expensive events, OS designers treat them very differently  from cache misses, which are routine and tolerated every cycle by hardware. This has direct tuning  implications: cache throughput is improved by better locality-exploiting hardware (associativity,  prefetching), while virtual-memory throughput is improved chiefly by minimizing fault rate  (adequate RAM, working-set-aware replacement, huge pages to cut TLB-miss overhead).  Conflating the two — for example, trying to fix a page-fault problem by resizing cache — would  be a category error.

# Synthesized Conclusion (Cross-Analysis Summary)

Combining the three views shows that cache and paging share a design principle but not a cost  structure or a management model. The structural lens explains the shared 'small-fast-in-front-of- large-slow' pattern; the quantitative lens shows the miss penalties differ by orders of magnitude  even though the hit-path latencies are of comparable order; and the applied lens shows this  asymmetry is exactly why the two are tuned with different levers. A sound system-level analysis  must treat cache and virtual-memory performance as related but independently governed.

# Question 4: NAND Flash vs DRAM

Analyze the differences between NAND Flash and DRAM with respect to latency, bandwidth, and suitability for hierarchical memory systems.

# Architecture Diagram

# Scope of Items Compared

The scope compares NAND Flash and DRAM as representative non-volatile-storage and volatile- main-memory technologies, on latency, bandwidth, and — critically for CO4 — their suitability  as a specific tier within a hierarchical memory system, since the two sit at very different points in  the latency/capacity/persistence trade-off space.

# Comparison Table

| Parameter | NAND Flash | DRAM |
| --- | --- | --- |
| Volatility | Non-volatile | Volatile refresh) (needs continuous |
| Read Latency | ~10-100 us (page read) | ~10-20 ns |
| Write Latency | Higher; requires block erase (~ms) | ~10-20 ns, no erase-before-write |
| Bandwidth | Hundreds of MB/s to few GB/s (NVMe) | Tens of GB/s (parallel channels) |
| Parameter | NAND Flash | DRAM |
| Endurance | Limited program/erase cycles | Effectively endurance unlimited write |
| Hierarchical Role | Persistent storage tier (SSD) — bulk/cold data | Main memory tier — active working set |

# Analysis 1 — Architectural / Structural Lens

NAND Flash stores charge in floating-gate or charge-trap cells that retain state without power, but  reading and especially writing require page- and block-level operations (erase-before-write) rather  than the direct, byte-addressable access DRAM supports. This structural constraint — block erase  — is what produces NAND's asymmetric read/write latency and its wear-out mechanism, neither  of which has an equivalent in DRAM's charge-refresh model.

# Analysis 2 — Quantitative / Data-Driven Lens

NAND Flash read latency (tens of microseconds) is roughly 1,000x slower than DRAM (tens of  nanoseconds), and NVMe SSD interface bandwidth, while high for storage, remains well below  DRAM channel bandwidth. This roughly three-orders-of-magnitude latency gap is consistent with  standard SSD/DRAM datasheet comparisons and is large enough that no amount of controller- level optimization closes it — it is a physical property of the storage medium, not an engineering  limitation.

# Analysis 3 — Application / Critical-Thinking Lens

Because the latency gap is so large, NAND cannot substitute for DRAM in any latency-sensitive  role; its value lies entirely in non-volatility and density/cost, making it suited to the storage tier  rather than the working-memory tier. In a hierarchical system this places NAND below DRAM,  with DRAM holding the active working set and NAND providing large, cheap, persistent capacity  — the same rationale behind DRAM-as-cache-for-flash patterns seen in OS page caches and SSD  DRAM buffers.

# Synthesized Conclusion (Cross-Analysis Summary)

The structural, quantitative, and applied perspectives all point to the same placement decision:  NAND's block-erase architecture produces a physical (not merely engineering) latency gap of  roughly three orders of magnitude relative to DRAM, and that gap is exactly why NAND occupies  the persistent-storage tier one level below DRAM rather than competing with it. Understanding  NAND vs DRAM therefore requires reading the cell-level mechanism, the measured latency gap,  and the tiering decision as one continuous argument.

# Question 5: MRAM vs RRAM

Compare MRAM and RRAM technologies by examining their performance in terms of access time, throughput, and energy efficiency.

# Architecture Diagram

# Scope of Items Compared

The scope compares MRAM (Magnetoresistive RAM) and RRAM (Resistive RAM), two leading  emerging non-volatile memory technologies positioned to bridge the gap between DRAM's speed  and Flash's non-volatility, evaluated on access time, throughput, and energy efficiency — the  dimensions most relevant to their candidacy as new tiers in next-generation hierarchical memory  systems.

# Comparison Table

| Parameter | MRAM | RRAM (ReRAM) |
| --- | --- | --- |
| Storage Mechanism | Magnetic tunnel junction (MTJ); spin orientation | Resistance change via conductive filament formation/rupture |
| Access Time (Read) | ~10-30 ns (near-DRAM) | ~10-50 ns (comparable, more variable) |
| Write Endurance | Very high (10^12-10^15 cycles) | Moderate (10^6-10^12 cycles) |
| Parameter | MRAM | RRAM (ReRAM) |
| Throughput | High, symmetric read/write | High read; write limited by filament variability |
| Energy Efficiency | Low read energy, moderate write energy | Very low read/write energy; suits compute-in-memory |
| Best Fit | Fast NV cache, frequent-write applications | Dense storage, in-memory AI compute |

# Analysis 1 — Architectural / Structural Lens

MRAM's magnetic tunnel junction stores information as electron spin orientation, a mechanism  that does not physically degrade the cell on each write. RRAM's metal-oxide dielectric instead  relies on forming and rupturing a conductive filament — a physically wearing process. This single  structural difference (non-degrading magnetic switching versus material-wearing filament  switching) is the root cause of nearly every performance gap between the two technologies.

# Analysis 2 — Quantitative / Data-Driven Lens

MRAM's endurance (10^12-10^15 cycles) is roughly 3-9 orders of magnitude higher than RRAM's  (10^6-10^12 cycles), directly reflecting the wear-versus-no-wear distinction. Access-time figures  are close (10-30 ns vs 10-50 ns, both near-DRAM class), which shows the two technologies are  speed-competitive even though their endurance and energy profiles diverge sharply — RRAM's  simpler two-terminal structure gives it a density and scaling advantage that partially offsets its  endurance disadvantage.

# Analysis 3 — Application / Critical-Thinking Lens

The endurance gap dictates suitability: MRAM's near-unlimited endurance makes it attractive  wherever data updates frequently (a persistent cache or working-register replacement), while  RRAM's lower endurance but smaller cell footprint and very low switching energy make it more  scalable and cost-effective for high-density, less write-intensive roles such as storing largely-static  AI model weights or supporting analog compute-in-memory, where data movement — not  computation — dominates energy consumption.

# Synthesized Conclusion (Cross-Analysis Summary)

Across all three lenses, the switching mechanism is the single variable that explains everything  else: it predicts the multi-order-of-magnitude endurance gap (quantitative) and, through that, the  differing application niches (applied) — MRAM for frequently-updated, latency-sensitive state,  RRAM for dense, largely-static, energy-efficient storage or compute-in-memory. A comparison  that reported access-time figures alone, without tracing them back to the switching mechanism,  would understate why the two technologies are not interchangeable despite their similar speed.