SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1-CONSTRAINT-BASED PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL1- Constraint-Based Problem Solving |
| Weightage: | 40% | Total Marks: | 25 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: | Nishevitha Sri.S | Reg.No.: | 192511424 |
| Department: | CSE | Date: |  |

ANNEXURE 	A

Constraint-Based 	Problem 	Solving  1.Construct an I/O communication mechanism for a real-time embedded system with the  following constraints:

- CPU 	utilization 	must 	be 	below 	15%

- Multiple 	sensors 	generate 	interrupts 	every 	1 	ms

- Use 	vectored 	interrupt 	handling

- Select between memory-mapped and I/O-mapped I/O for optimal latency

2.Design an I/O subsystem under these constraints:

- High-speed storage requires throughput > 3 GB/s

- Must 	use 	NVMe 	over 	PCIe

- CPU should not handle bulk data transfer

- Integrate DMA controller with interrupt notification

3.Construct a multi-device communication architecture with constraints:

- 6 peripherals using USB, 2 high-speed devices using Thunderbolt

- Avoid 	interrupt 	starvation

- Use 	hardware 	and 	software 	interrupts

- Ensure fair bus arbitration

4.Design a data acquisition system with constraints:

- Continuous data stream at 500 MB/s  ● Minimal CPU intervention required

- Must compare DMA transfer vs interrupt-driven I/O

- Use memory-mapped I/O for device registers

5.Construct an interrupt handling mechanism with constraints:

- Support 	nested 	interrupts

- Maximum interrupt latency < 10 µs

# ANNEXURE A

Below are exam-ready answers for all 5 questions. I’ve written them in a clear constraint → design → working → justification format.

ANNEXURE A – Constraint-Based Problem Solving

1. I/O Communication Mechanism for a Real-Time Embedded System

## Constraints

- CPU utilization must be below 15%.

- Multiple sensors generate interrupts every 1 ms.

- Use vectored interrupt handling.

- Select between memory-mapped I/O and I/O-mapped I/O for optimal latency.

## Solution

A vectored interrupt-based memory-mapped I/O mechanism is suitable for the real-time embedded system. Each sensor is connected to an interrupt controller, and every sensor is assigned a unique interrupt vector.

## Architecture

## Working

- Sensors continuously monitor physical parameters.

- When a sensor requires CPU attention, it generates an interrupt.

- The interrupt controller identifies the source.

- The corresponding interrupt vector directly points to the appropriate ISR.

- The CPU executes the ISR and reads the sensor register.

- After servicing, the CPU returns to the previous task.

- DMA can be used for larger sensor data transfers to further reduce CPU utilization. Memory-Mapped vs I/O-Mapped I/O

I/OMemory-Mapped

Feature 	Mapped

## I/O

I/O

Address 	Shared 	with 	Separate 	I/O space 	memory 	address space

Normal 	memory 	Special 	I/O

Instructions

instructions 	instructions

Programming 	Easier 	More specialized

Low, 	but

Latency 	Low 	architecture-

dependent

Suitable here 	Yes 	Less preferred

## Justification

Memory-mapped I/O is selected because it provides simple and fast access to sensor registers and integrates well with the processor's normal load/store operations. Vectored interrupts reduce interruptidentification overhead. DMA can be added for bulk data transfer.

## Conclusion

The proposed system achieves low interrupt latency and keeps CPU utilization below 15% by combining memory-mapped I/O, vectored interrupts, interrupt prioritization, and DMA.

2. I/O Subsystem for High-Speed NVMe Storage

## Constraints

- Throughput must be greater than 3 GB/s.

- Must use NVMe over PCIe.

- CPU should not handle bulk data transfer.

- DMA controller must be integrated with interrupt notification.

## Solution

Use an NVMe SSD connected through PCIe, with DMA-based data transfer between the SSD and main memory.

Architecture

## Working

- CPU sends an NVMe command through the NVMe driver.

- The NVMe controller processes the command.

- Data is transferred directly between the NVMe device and RAM using DMA.

- CPU does not copy the bulk data.

- After completing the transfer, the NVMe controller generates an interrupt.

- The CPU handles the completion notification.

- NVMe queues allow multiple commands to be processed efficiently.

Why DMA?

Without DMA:

NVMe → CPU → Memory

The CPU would spend significant time moving data.

With DMA:

NVMe → DMA → Memory

The CPU only manages commands and completion notifications.

## Throughput

PCIe provides high-bandwidth communication, while NVMe is designed for high-speed storage and parallel queues. The design should use an appropriate PCIe generation/lane configuration capable of sustaining >3 GB/s in practice.

## Conclusion

The optimal subsystem is NVMe + PCIe + DMA + interrupt notification. This achieves high throughput while keeping CPU involvement low.

3. Multi-Device Communication Architecture

## Constraints

- 6 USB peripherals.

- 2 high-speed Thunderbolt devices.

- Avoid interrupt starvation.

- Use hardware and software interrupts.

- Ensure fair bus arbitration.

## Architecture

## Working

- Six USB devices communicate through the USB host controller.

- Two high-speed devices communicate through Thunderbolt.

- Hardware interrupts are generated by devices requiring immediate attention.

- Software interrupts are used for OS services and deferred/background processing.

- An interrupt controller assigns priorities to interrupts.

- High-priority interrupts are serviced first.

- Lower-priority interrupts are placed in queues instead of being permanently blocked.

- Fair bus arbitration allocates communication opportunities among devices. Avoiding Interrupt Starvation Use:

- Priority-based interrupt handling.

- Round-robin service for devices of equal priority.

- Interrupt masking only for very short critical sections.

- Interrupt queues.

- Deferred interrupt processing for lengthy operations.

## Fair Bus Arbitration

A round-robin arbitration mechanism can be used:

USB1 → USB2 → USB3 → USB4 → USB5 → USB6

↑                                      ↓

+------------- Repeat ----------------+

Thunderbolt devices can receive higher bandwidth allocations when required, while arbitration prevents any device from monopolizing the shared resources.

## Conclusion

The architecture supports 6 USB + 2 Thunderbolt devices while preventing interrupt starvation through priorities, queues, round-robin arbitration, hardware interrupts, and software interrupts.

4. Data Acquisition System – 500 MB/s

## Constraints

- Continuous data stream of 500 MB/s.

- Minimal CPU intervention.

- Compare DMA transfer vs interrupt-driven I/O.

- Use memory-mapped I/O for device registers.

## Architecture

## Working

- The data acquisition device continuously generates data.

- Its control/status registers are accessed using memory-mapped I/O.

- DMA is configured with:

o 	Source address o 	Destination address o 	Transfer size

- The device transfers data directly to memory.

- DMA generates an interrupt after a block transfer is completed.

- CPU processes the completed buffer while DMA fills another buffer.

## Double Buffering

Device

|   v

+---------+       +---------+

| Buffer A|       | Buffer B|

+---------+       +---------+

↑                 ↑

DMA              CPU

While DMA fills Buffer A, the CPU processes Buffer B. Then they switch.

DMA vs Interrupt-Driven I/O

## Parameter 	DMA 	Interrupt-Driven I/O

CPU involvement 	Very low 	High Suitable for 500 MB/s 	Yes 	No

Device 	↔ 	CPU 	↔

Data movement 	Direct device ↔ memory

memory

Potentially

Interrupt frequency 	Block-based

very high

CPU overhead 	Low 	High

Performance 	High 	Lower

## Why DMA is Better

At 500 MB/s, interrupt-driven I/O would require frequent CPU intervention and could consume significant processing time. DMA transfers large blocks directly to memory and interrupts the CPU only when necessary.

## Conclusion

The best solution is memory-mapped I/O + DMA + double buffering. It provides continuous 500 MB/s acquisition with minimal CPU intervention.

5. Interrupt Handling Mechanism

## Constraints

- Support nested interrupts.

- Maximum interrupt latency < 10 µs.

- Use vectored interrupts for high-priority devices.

- Use software interrupts for background tasks.

## Architecture

## Working

- A device generates an interrupt.

- The interrupt controller determines its priority.

- A high-priority device receives a vectored interrupt.

- CPU immediately jumps to its corresponding ISR.

- If another higher-priority interrupt occurs during ISR execution, the current ISR can be temporarily suspended.

- CPU saves the current context and services the higher-priority interrupt.

- After completion, the previous ISR resumes.

- Software interrupts are used for deferred/background processing.

## Nested Interrupt Example

Main Program

|      v

Low Priority ISR

|

|---- High Priority Interrupt

|          |

|          v

|    High Priority ISR

|          |

|          v

|    Return to Low ISR

|      v

Main Program

## Meeting the <10 µs Requirement

To minimize interrupt latency:

- Use vectored interrupts.

- Keep ISRs short.

- Use a fast interrupt controller.

- Use priority levels.

- Avoid unnecessary interrupt masking.

- Move lengthy processing to deferred/software interrupt handlers.

- Use DMA for large data transfers.

## Priority Structure

Priority 1 → Emergency / Real-Time Device

Priority 2 → High-Speed I/O

Priority 3 → Normal I/O

Priority 4 → Background Tasks

## Conclusion

A priority-based nested interrupt system with vectored interrupts provides fast response for critical devices. Software interrupts handle non-critical background tasks, while short ISRs help maintain the required <10 µs maximum interrupt latency.

Quick Final Summary

## Question 	Best Design

1 	Memory-mapped I/O + vectored interrupts + DMA 2 	NVMe + PCIe + DMA + completion interrupts

USB + Thunderbolt + priority interrupts + round-

3

robin arbitration

- Memory-mapped I/O + DMA + double buffering

Nested vectored interrupts + priority controller +

- software interrupts